COUNTY; ROUTE; PROJECT #; ITEM #. 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; 16. Inlet/ MH (ID); Station; Area, A; “C”; CA; Σ CA; T (min); Σ T (min); I __ (in/hr); Q (cfs); Pipe Length (LF); Pipe Slope (ft/ft); Pipe Size (in); Mean Vel (fps); Full Flow (cfs); Cap (%).

INSTRUCTIONS COL #; ITEM; DESCRIPTION. 1.; End of Pipe ID; Inlet, manhole or junction number of symbol. 2.; Station; Roadway station or end of pipe location. 3.; Drainage area, A; Contributing drainage area at inlet or manhole. 4.; Runoff coefficient, C; Representative runoff coefficient of drainage area. 5.; CA; Intermediate runoff calculation. 6.; ∑CA; Summation of CAs to this point. 7.; Pipe travel time, T; Pipe length (col. 11)/Mean Velocity (col. 14). 8.; Total travel time; Travel time to inlet (col. 8) + pipe travel time (col. 7). 9.; Rainfall intensity, I; Compute based on total travel time (col. 8). 10.; Flow, Q; Rational discharge = CA (col. 6) x I (col. 9). 11.; Pipe Length, L; Place pipe entries between inlets, manholes, or junctions. 12.; Pipe Slope, So; Slope of pipe in ft/ft. 13.; Pipe diameter, D; Determine from nomograph or other means (n=0.012). 14.; Mean pipe velocity, V; Determine from nomograph or other means. 15.; Full pipe flow; Determine from nomograph or other means. 16.; Capacity (%); Design capacity = Flow (col. 10)/Full pipe flow (col. 15); If design flow produces pressure flow, resize pipe..