General Notes

DIVISION 100 -- GENERAL PROVISIONS

USE THE FOLLOWING NOTE ON PROJECTS INVOLVING N.G.S. (U.S.G.S.) BENCH MARKS.

160	N.G.S. (U.S.G.S.) BENCH MARKS

DO NOT DISTURB N.G.S. (U.S.G.S.) BENCH MARKS IN ANY MANNER UNLESS DIRECTED BY THE ENGINEER.

USE THE FOLLOWING NOTE ON ALL PROJECTS.  (DESIGN MEMO 1A-86)

165	BEFORE YOU DIG

The contractor is instructed to call 1-800-752-6007 to reach KY 811, the one-call system for information on the location of existing underground utilities.  The call is to be placed a minimum of two (2) and no more than ten (10) business days prior to excavation.  The contractor should be aware that owners of underground facilities are not required to be members of the KY 811 one-call Before-U-Dig (BUD) service.  The contractor must coordinate excavation with the utility owners, including those whom do not subscribe to KY 811.  It may be necessary for the contractor to contact the County Court Clerk to determine what utility companies have facilities in the area.

USE THE FOLLOWING NOTE ON ALL PROJECTS, EXCEPT WHEN OTHERWISE DIRECTED BY THE PERMITS COORDINATOR.  (DESIGN MEMO 3A-84)  (REVISED BY MEMO FROM RICKY YOUNG DATED 5-12-93)

190	DEPARTMENT OF THE ARMY PERMIT AND WATER QUALITY CERTIFICATION APPROVALS

A DEPARTMENT OF THE ARMY (DA) PERMIT, WHICH MAY REQUIRE APPROVAL OF A STATE WATER QUALITY CERTIFICATION FROM THE KENTUCKY DIVISION OF WATER, REGULATES THIS PROJECT AT ONE OR MORE LOCATIONS.  PERFORM ALL APPLICABLE WORK IN COMPLIANCE WITH THE CONDITIONS STATED IN THE DA PERMIT AND THE APPROVED WATER QUALITY CERTIFICATION.  POST A COPY OF THE DA PERMIT AND THE WATER QUALITY CERTIFICATION IN A CONSPICUOUS PLACE AT THE PROJECT SITE.  IF A DA PERMIT OR WATER QUALITY CERTIFICATION APPROVAL IS PENDING, DO NOT WORK IN OR DISTURB THE DESIGNATED AREA(S) UNTIL OBTAINING THE APPROPRIATE APPROVAL(S).  REFER TO NOTICE(S) CONTAINED IN THE CONTRACT BID PROPOSAL FOR DESIGNATED AREA(S) WHERE WORK IS PROHIBITED BY THE ABSENCE OF APPROVAL.

DIVISION 200 -- EARTHWORK

USE THE FOLLOWING NOTE ON PROJECTS WHERE WE DO NOT ANTICIPATE PAYING FOR "CLEARING AND GRUBBING".

200	CLEARING AND GRUBBING

CONTRARY TO SECTION 202 OF THE STANDARD SPECIFICATIONS, NO DIRECT PAYMENT WILL BE ALLOWED FOR CLEARING AND GRUBBING ON THIS PROJECT.

USE THE FOLLOWING NOTE ON ALL PROJECTS WHICH REQUIRE ADJUSTMENTS TO EARTHWORK QUANTITIES BECAUSE OF CHANGING THE PAVEMENT THICKNESS.  ALSO, SHOW THE AMOUNT OF THE ADJUSTMENT SEPARATELY IN THE EARTHWORK CALCULATIONS.  (RPR MEMO 1-83)

## 231	DESIGN CHANGE

CROSS-SECTIONS FOR THIS PROJECT WERE DEVELOPED FOR A PAVEMENT THICKNESS OF ? INCHES.  THE PAVEMENT DESIGN WAS LATER REVISED.  TYPICAL SECTIONS WERE AND EARTHWORK QUANTITIES WERE ADJUSTED TO REFLECT THIS CHANGE.   HOWEVER, CROSS-SECTIONS WERE NOT REVISED AND, IN THIS CASE, TYPICAL SECTIONS GOVERN OVER CROSS-SECTIONS.  MAKE FINAL PAYMENT ON ADJUSTED QUANTITIES.

USE THE FOLLOWING NOTE ON ALL GRADE, DRAIN AND SURFACING PROJECTS REQUIRING ADJUSTMENTS TO EARTHWORK QUANTITIES FOR ALTERNATE PAVEMENT DESIGNS.  DO NOT USE PLAN NOTE NO. 231 FOR THIS PURPOSE.  ALSO, SHOW THE AMOUNT ADJUSTED SEPARATELY IN THE EARTHWORK CALCULATIONS. (RPR MEMO 2-90)

233	EARTHWORK ADJUSTMENTS FOR ALTERNATE PAVEMENT DESIGNS

CROSS-SECTIONS FOR THIS PROJECT WERE DEVELOPED FOR A PAVEMENT THICKNESS OF ? INCHES (MM).  INCLUDED IS TYPICAL SECTIONS FOR ALTERNATE PAVEMENT DESIGNS AND ADJUSTED EARTHWORK QUANTITIES TO REFLECT THESE ALTERNATES.  HOWEVER, THERE IS NO DEVELOPED CROSS-SECTIONS FOR ALL ALTERNATIVES SO TYPICAL SECTIONS GOVERN OVER CROSS-SECTIONS.  MAKE FINAL PAYMENT ON ADJUSTED QUANTITIES.

USE THE FOLLOWING NOTE WHEN:

(1)	SPECIFYING BANK GRAVEL ROADBED DUE TO ITS ABUNDANCE IN THE

SURROUNDING AREA.

(2)	SOILS PROFILE IS IN PLANS.

246	SELECTED BANK GRAVEL ROADBED NOTE

CONSTRUCT THE TOP 12 INCHES (300 MM) OF THE ENTIRE ROADBED, AND SHOULDER PORTION INDICATED ON THE TYPICAL SECTION, OF ROADWAY EXCAVATION AND/OR BORROW EXCAVATION COMPOSED OF BANK GRAVEL THAT CONTAINS NOT LESS THAN 35% MATERIAL RETAINED ON THE 4.75 MM (NO. 4) SIEVE.  WHEN ENCOUNTERING THIS MATERIAL AT ROADBED ELEVATION, DO NOT UNDERCUT BUT COMPACT IN PLACE IN ACCORDANCE WITH THE STANDARD SPECIFICATIONS.

USE THE FOLLOWING NOTE WHEN:

(1)	SPECIFYING "EMBANKMENT-IN-PLACE".

(2)	SPECIFYING A C.B.R. VALUE (MATERIALS MEMO).

(3)	LONGER PROJECTS, MAY OR MAY NOT HAVE A SOILS PROFILE.

BRING TO THE ATTENTION OF THE PAVEMENT DESIGN SECTION WHEN THERE IS AN APPARENT DISCREPANCY BETWEEN THE ROADBED NOTE ISSUED WITH THE PAVEMENT DESIGN AND THE WARRANTS DESCRIBED HEREIN FOR NOTES (241) THROUGH (248).

248	ROADBED NOTE, EMBANKMENT-IN-PLACE

USE EMBANKMENT-IN-PLACE WITH A MINIMUM C.B.R. VALUE OF ? OR GREATER IN THE TOP 12 INCHES (300 MM) OF ROADBED IN CUT AND FILL SECTIONS.

USE THE FOLLOWING NOTES, AS APPLICABLE, WHEN THE PAVEMENT DESIGN SPECIFIES. INSERT THE SPECIFIED ROADBED THICKNESS, THE C.B.R. VALUE, THE REQUIRED PERCENTAGE OF STABILIZATION MATERIAL AND THE AVERAGE DRY DENSITY IN THE PROVIDED BLANKS.  (RPR MEMOS 2-87 AND 1-90) (REVISED BY G. W. SHARPE 9-17-91 TO INCLUDE C.B.R. VALUE)

275	CEMENT STABILIZED ROADBED

STABILIZE THE TOP ? INCHES (MM) OF THE FINISHED ROADBED WITH PORTLAND CEMENT IN ACCORDANCE WITH SECTION 208 OF THE STANDARD SPECIFICATIONS.  USE SELECTED SOILS, WITH A MINIMUM CBR VALUE OF ? , FOR THIS PURPOSE.

THE PORTLAND CEMENT CONTENT IS ? PERCENT BY WEIGHT, AND THE ESTIMATED PLAN QUANTITY USES AN AVERAGE DRY DENSITY OF ? LBS/CUBIC FEET (KG/CUBIC METER). HOWEVER, ADJUST THE QUANTITY AFTER CONSTRUCTING THE ROADBED AND SUBMITTING THE SAMPLES FOR TESTING.  THIS TAKES APPROXIMATELY TWO WEEKS.

276	LIME STABILIZED ROADBED

STABILIZE THE TOP ? INCHES (MM) OF THE FINISHED ROADBED WITH LIME IN ACCORDANCE WITH SECTION 208 OF THE STANDARD SPECIFICATIONS.  USE SELECTED SOILS, HAVING A MINIMUM C.B.R. VALUE OF ? FOR THIS PURPOSE.

THE LIME CONTENT IS ? PERCENT BY WEIGHT, AND THE ESTIMATED PLAN QUANTITY USES AN AVERAGE DRY DENSITY OF ? LBS/CUBIC FEET (KG/CUBIC METER).  HOWEVER, ADJUST THE QUANTITY AFTER CONSTRUCTING THE ROADBED AND SUBMITTING THE SAMPLES FOR TESTING.  THIS TAKES APPROXIMATELY TWO WEEKS.

USE THE FOLLOWING NOTE ON GRADE AND DRAIN PROJECTS WITH A CHEMICALLY STABILIZED ROADBED AND WHEN THE SURFACE IS CONSTRUCTED AS PART OF A SEPARATE CONTRACT.  (GEOTECH.-STEVEN HENDERSON 5/01)

278	GRADE AND DRAIN ONLY WITH CHEMICALLY STABILIZED ROADBED

THIS CONTRACT IS FOR GRADE AND DRAIN CONSTRUCTION ONLY, BUT THE SURFACE CONTRACT WILL REQUIRE A CHEMICALLY STABILIZED ROADBED.  PREPARE THE SUBGRADE IN ACCORDANCE WITH SUBSECTION 208.03 OF THE STANDARD SPECIFICATIONS.

CONSTRUCT ROADBED IN ACCORDANCE WITH SUBSECTION 208.03 OF THE STANDARD SPECIFICATIONS.

DIVISION 400 -- ASPHALT PAVEMENTS

USE THE FOLLOWING NOTE ON 4-R ASPHALT OVERLAY PROJECTS WHICH EXTEND THROUGH THE WINTER CLOSEDOWN PERIOD.  (RPR MEMO 4-87)

429	WINTER CLOSEDOWN

ANY ASPHALT CONCRETE BASE AND/OR SURFACE COURSE USED AS A RIDING SURFACE EXPOSED TO TRAFFIC DURING WINTER CLOSEDOWN PERIODS SHALL CONTAIN NATURAL, CONGLOMERATE, CRUSHED SLAG, CRUSHED GRANITE OR CRUSHED SANDSTONE SAND IN THE PROPORTION OF NO LESS THAN 25% OF THE TOTAL COMBINED COARSE AND FINE AGGREATE.

USE THE FOLLOWING NOTE ON ALL PROJECTS WITH A BID ITEM FOR "BREAKING AND SEATING PAVEMENT".  (RPR MEMO 3-88)

435	PATCHING REMOVAL

REMOVE ALL ASPHALT PATCHING MATERIAL PRIOR TO BREAKING AND SEATING.  THE MATERIALS REMOVED SHALL BECOME THE PROPERTY OF THE CONTRACTOR.  THE COST OF REMOVAL AND DISPOSAL OF PATCHING MATERIALS IS INCIDENTAL TO "BREAKING AND SEATING PAVEMENT".

USE PLAN NOTE 555 ON PROJECTS WITH RIGID (JPC) PAVEMENTS.

USE THE FOLLOWING NOTE ON PROJECTS WITH AT LEAST 0.4 MILE (0.64 KM) OR GREATER OF SURFACING MEETING ANY OF THE FOLLOWING CRITERIA:

1.	NEW CONSTRUCTION PROJECTS REGARDLESS OF POSTED SPEED LIMIT.

2.	SURFACING PROJECTS INVOLVING TWO OR MORE ASPHALT COURSES ON HIGH-TYPE FACILITIES AND A POSTED SPEED OF 45 MPH OR GREATER.

3.	SURFACING PROJECTS ON FACILITIES WITH A POSTED SPEED OF 45 MPH OR GREATER WHERE SIGNIFICANT MILLING OR OTHER SURFACE PREPARATION MAKES RIDEABLILTY REQUIREMENTS FEASIBLE.

THE DEPARTMENT MAY APPLY RIDEABILITY ON PROJECT TYPES OTHER THAN THE ABOVE OR WAIVE THE REQUIREMENTS WHEN DEEMED NECESSARY.

CATEGORY.  ONCE A DETERMINATION HAS BEEN MADE THAT RIDEABILITY REQUIREMENTS WILL BE APPLIED, THE DEPARTMENT WILL ESTABLISH THE PAY ADJUSTMENT SCHEDULE TO BE USED BASED ON THE GUIDELINES BELOW.

CATEGORY “A” PAY ADJUSTMENT SCHEDULE WILL BE USED UNLESS ANY OF THE FOLLOWING CONDITIONS EXIST:

1.          	THE ROUTE HAS A POSTED SPEED LIMIT OF 45 MPH OR LESS.

2.	MAINTENANCE OF TRAFFIC REQUIREMENTS OR OTHER CONSTRUCTION ISSUES WILL RESULT

IN EXCESSIVE DISCONTINUITIES IN THE SURFACE COURSE.

3.	THE PROJECT ONLY INVOLVES A SINGLE ASPHALT COURSE.

4.	THE SURFACE TYPE IS CLASSIFIED AS “EXPERIMENTAL”.

5.	OTHER CIRCUMSTANCES INDICATE THAT MORE STRINGENT RIDE QUALITY SPECIFICATIONS WOULD NOT BE ATTAINABLE OR ARE NOT IN THE BEST INTEREST OF THE CABINET.

THE CATEGORY “B” PAY ADJUSTMENT SCHEDULE WILL APPLY IF ANY OF THE ABOVE CONDITIONS ARE

PRESENT.

COORDINATE THE APPLICATION OF THIS NOTE, ESPECIALLY IN UNUSUAL CIRCUMSTANCES,

WITH THE PAVEMENT MANAGEMENT STAFF REVISED WARRANTS BY LETTER DATED 2-13-08 FROM STATE HIGHWAY ENGINEER O. GILBERT NEWMAN – APPROVAL FROM FHWA – JOSE SEPULVEDA LETTER DATED 2-27-08.

444	ASPHALT PAVEMENT RIDE QUALITY

PAVEMENT RIDEABILITY REQUIREMENTS, IN ACCORDANCE WITH SECTION 410 OF THE STANDARD SPECIFICATIONS, SHALL APPLY ON THIS PROJECT.  CATEGORY ? SHALL APPLY.

USE ONE OF THE FOLLOWING NOTES AS DIRECTED BY THE PAVEMENT DESIGN.

447	COMPACTION OF ASPHALT MIXTURES

WILL ACCEPT THE COMPACTION OF ASPHALT MIXTURES FURNISHED FOR DRIVING LANES AND RAMPS AT ONE INCH (25 MM) OR GREATER ON THIS PROJECT BY OPTION A ACCORDING TO SUBSECTIONS 402 AND 403 OF THE CURRENT STANDARD SPECIFICATIONS.  USE JOINT CORES AS DESCRIBED IN SUBSECTION 402.03.02 FOR SURFACE MIXTURES ONLY.  WILL ACCEPT THE COMPACTION OF ALL OTHER ASPHALT MIXTURES BY OPTION B.

448	COMPACTION OF ASPHALT MIXTURES

WILL ACCEPT THE COMPACTION OF ASPHALT MIXTURES FURNISHED ON THIS PROJECT BY OPTION B ACCORDING TO SUBSECTIONS 402.03.02 AND 403.03.10 OF THE STANDARD SPECIFICATIONS.

USE THE FOLLOWING NOTE IN CONJUNCTION WITH A DETAIL ON PROJECTS HAVING THE BID ITEM OF "EDGE KEY".  (RPR MEMO 1-84)

455	EDGE KEY

THIS WORK INCLUDES CUTTING OUT THE EXISTING ASPHALT SURFACE TO A MINIMUM DEPTH AND WIDTH AS DETAILED ELSEWHERE IN THE PLANS SO THAT THE NEW SURFACE MAY HEEL INTO THE EXISTING SURFACE.  THE CONTRACT UNIT PRICE BID LINEAR FOOT (PER METER) FOR "EDGE KEY" INCLUDES ALL NECESSARY MATERIALS, LABOR AND EQUIPMENT NECESSARY TO PERFORM THE WORK AND DISPOSE OF THE REMOVED ASPHALT MATERIAL.

DIVISION 500 - JPC PAVEMENT AND NON-STRUCTURAL CONCRETE CONSTRUCTION

USE PLAN NOTE 444 ON PROJECTS WITH ASPHALT PAVEMENTS.

USE THE FOLLOWING NOTE ON PROJECTS WITH RIGID (JPC) PAVEMENTS MEETING THE

FOLLOWING CRITERIA:

1.	ALL NEW CONSTRUCTION PROJECTS WITH AN EXPECTED POSTED SPEED OF 45 MPH OR GREATER, AND WITH AT LEAST 0.4 MILE (0.64 KM) OF SURFACING UNINTERRUPTED BY BRIDGES OR STOPS AT INTERSECTIONS.

2.	ALL INTERSTATE OR TOLL ROAD PROJECTS.

3.	OTHER HIGH-TYPE, HIGH SPEED HIGHWAYS WHEN THE CONDITIONS OF THE EXISTING PAVEMENT, OR OTHER SPECIAL CIRCUMSTANCES, MAKE APPLICATION OF JPC PAVEMENT SMOOTHNESS REQUIREMENTS FEASIBLE OR DESIRABLE.

COORDINATE THE APPLICATION OF THIS NOTE, ESPECIALLY IN UNUSUAL CIRCUMSTANCES, WITH THE PAVEMENT MANAGEMENT STAFF.  (MEMO FROM E. B. DRAKE DATED 1-27-87)

555	JPC PAVEMENT RIDE QUALITY

APPLY JPC PAVEMENT SMOOTHNESS REQUIREMENTS, IN ACCORDANCE WITH SUBSECTION 501.03.19 OF THE STANDARD SPECIFICATIONS, ON THIS PROJECT.

DIVISION 600 -- STRUCTURES AND CONCRETE

USE THE FOLLOWING NOTE WHEN SPECIFYING PRECAST BOX CULVERTS WITH NO CAST-IN-PLACE ALTERNATE.  ALSO, INCLUDE A QUANTITY FOR "STRUCTURE EXCAVATION". (G.C.D. MEMO DATED 3-12-91)

641	PRECAST REINFORCED CONCRETE BOX SECTIONS

CONTRARY TO STANDARD DRAWING NO. RDI-100, ASSOCIATE HS 25 LOADING WITH TABLE 1-4.  MODIFY THE REQUIRED STEEL TO REFLECT THE GREATER LIVE LOADING. SEE THE APPENDICES IN ASTM C789 AND ASTM C850.  THE CALCULATIONS FOR THIS GREATER LIVE LOAD MAY CHANGE THE TABLE NO. DEPICTED ON THE DESIGN PLANS.

USE THE FOLLOWING NOTE ON ALL PLANS – REV. 1/24/2006 – DAVE KRATT, ANDRE JOHANNES, BILL GREENE

650	STANDARD DRAWINGS

STANDARD DRAWINGS ARE NOT ATTACHED TO THESE PLANS.  A STANDARD DRAWING BOOK AND THE HEADWALL SUPPLEMENTAL BOOK MAY BE OBTAINED FROM THE POLICY SUPPORT BRANCH OF THE DEPARTMENT OF ADMINISTRATIVE SERVICES IN FRANKFORT, KY. AT (502) 564-4610

DIVISION 700 -- DRAINAGE, TRAFFIC AND ROADSIDE CONSTRUCTION

PLACE THE FOLLOWING NOTE ON PREVIOUSLY LANDSCAPED PROJECTS.

730	EXISTING LANDSCAPE MATERIAL

DO NOT DISTURB LANDSCAPE PLANTS UNLESS ABSOLUTELY NECESSARY.

DO NOT TO DISTURB ANY PLANTS WITHOUT PRIOR APPROVAL OF THE PROJECT ENGINEER.

THE PROJECT ENGINEER SHALL NOTIFY THE DISTRICT AGRONOMIST FOR POSSIBLE DEPARTMENT SALVAGE OF ANY PLANTS NEEDING REMOVAL.

INDEX TO NOTES

NOT CODED FOR THE COMPUTER

DIVISION 100 - GENERAL PROVISIONS

NOTE NO.

MOBILIZATION AND DEOMOBILIZATION	1

DIVISION 200 - EARTHWORK

CLEARING AND GRUBBING	1, 2

EMBANKMENT BENCH QUANTITIES	3, 4

SHRINK AND SWELL FACTORS	5

REMOVAL ITEMS (WITH SALVAGE VALUE TO THE DEPARTMENT)	6

TYPICAL SECTION NOTE	7

WATER	8

DIVISION 300 - AGGREGATE BASE COURSES

ADDITIONAL PAVEMENT QUANTITIES (ROCK ROADBED)	1, 2

DIVISION 700 - DRAINAGE, TRAFFIC AND ROADSIDE CONSTRUCTION

GUARDRAIL	1, 2

DIVISION 400 - ASPHALT PAVEMENTS

ASPHALT BASE (SLAG)	1, 2

DIVISION 700 - DRAINAGE, TRAFFIC AND ROADSIDE CONSTRUCTION

ADJUSTING DRAINAGE STRUCTURES	1

GUARDRAIL	2, 3

UNCODED NOTES FOR PLOTTING

THE FOLLOWING NOTES ARE EITHER FOR SPECIALIZED APPLICATION, REFERENCE NOTES,

OR RARE NOTES, INCONVENIENT FOR PLOTTING.

### DIVISION 100 - GENERAL PROVISION

MOBILIZATION AND DEMOBILIZATION

BID DEMOBILIZATION ON ALL PROJECTS.  BID MOBILIZATION ON ALL PROJECTS WITH AN

ESTABLISHED CONSTRUCTION COST IN EXCESS OF $2,000,000.  (SPECIAL PROVISION NO.

114 ISSUED BY DESIGN MEMO 4-11-75)

WHEN COMBINING TWO OR MORE SETS OF PLANS IN THE PROPOSAL INTO ONE CONTRACT, OR

WHEN COMBINING TWO OR MORE FEDERAL PROJECTS INTO ONE SET OF PLANS, BID

"MOBILIZATION" AND "DEMOBILIZATION" ON THE LARGER SET OF PLANS (OR PROJECT) AND

REFERENCE THE FOLLOWING NOTE:

(1)	"THE LUMP SUM BID ON THIS PROJECT SHALL INCLUDE PAYMENT IN FULL FOR         MOBILIZATION AND DEMOBILIZATION ON (COMPANION PROJECTS)."

### DIVISION 200 - EARTHWORK

CLEARING AND GRUBBING

WHEN "CLEARING AND GRUBBING" IS BID LUMP SUM IN ACCORDANCE WITH THE STANDARD

SPECIFICATIONS, REFERENCE ONE OF THE FOLLOWING NOTES TO THE BID ITEM:

- "APPROXIMATELY ______ ACRES."

OR

(2)	"APPROXIMATELY ______ HECTARES."

EMBANKMENT BENCH QUANTITIES

ON PROJECTS WITH EMBANKMENT BENCHING, SHOW THE QUANTITIES IN THE SHEET TOTALS.

BRING THE QUANTITIES TO THE GENERAL SUMMARY AND INCLUDE THEM IN THE ROADWAY

EXCAVATION OR EMBANKMENT-IN-PLACE QUANTITIES, AS APPLICABLE.  REFERENCE THE

TOTALS AND/OR SUB-TOTALS WITH ONE OF THE FOLLOWING NOTES:

(3)	"INCLUDES ______ CUBIC METERS FOR EMBANKMENT BENCHING."

OR

(4)	"INCLUDES ______ CUBIC YARDS FOR EMBANKMENT BENCHING."

SHRINKAGE AND SWELL FACTORS

REMOVAL ITEMS

(WITH SALVAGE VALUE TO THE DEPARTMENT)

REFERENCE THE FOLLOWING NOTE TO THE BID ITEM WHEN SALVAGING REMOVAL ITEMS FOR

THE DEPARTMENT'S USE:

(6)	"DISMANTLE, SALVAGE, AND STOCKPILE ON THE RIGHT-OF-WAY AS DIRECTED BY THE ENGINEER.  SALVAGED ITEMS SHALL BECOME THE PROPERTY OF THE DEPARTMENT OF HIGHWAYS".

TYPICAL SECTION NOTE

PLACE THE FOLLOWING NOTE ON THE TYPICAL SECTION SHEETS ON ALL PROJECTS:

(7)	"SEE CROSS-SECTIONS FOR SLOPES OUTSIDE THE LIMITS OF THE SHOULDER."

WATER

WHEN BIDDING WATER FOR CONTROLLING DUST CAUSED BY MAINTAINING TRAFFIC, IN

ACCORDANCE WITH SUBSECTION 107.01.04 OF THE STANDARD SPECIFICATIONS, REFERENCE

THE BID ITEM TO THE FOLLOWING NOTE (REVOSED AS DIRECTED BY C.D. POWERS

(11-29-89):

(8)	"FOR CONTROLLING DUST CAUSED BY MAINTAINING TRAFFIC ONLY."

SEE DESIGN MEMO 10-72 (DATED 5-4-72) FOR METHOD OF ESTIMATING WATER FOR DUST

CONTROL.

### DIVISION 300 - AGGREGATE BASE COURSES

ADDITIONAL PAVEMENT QUANTITIES

(ROCK ROADBED)

ON PROJECTS WITH ROCK ROADBED, ADD 10% TO THE BOTTOM 4" (100 MM) OF THE

PAVEMENT DESIGN AND REFERENCE ONE OF THE FOLLOWING NOTES TO THE ADJUSTED

QUANTITIES:

(1)	"INCLUDES 10% ADDITIONAL MATERIAL IN THE BOTTOM 4" OF THE PAVEMENT DESIGN AS A CONSTRUCTION TOLERANCE ON ROCK SUBGRADE."

OR

(2)	"INCLUDES 10% ADDITIONAL MATERIAL IN THE BOTTOM 100 MM OF THE PAVEMENT DESIGN AS A CONSTRUCTION TOLERANCE ON ROCK SUBGRADE."

### DIVISION 400 - ASPHALT PAVEMENTS

ASPHALT BASE (SLAG)

ON PROJECTS IN THE FOUR SLAG COUNTIES (BOYD, GREENUP, LAWRENCE AND LEWIS) WITH

THE BID ITEM ASPHALT BASE AND/OR ASPHALT BINDER, USE ONE OF THE FOLLOWING

REFERENCE NOTES:

(1)	"MAY UTILIZE BLAST FURNACE SLAG IN AN AGGREGRATE BLEND IN THIS ITEM IN ACCORDANCE WITH APPROVED MIX DESIGNS AND CURRENT SPECIFICATIONS. ESTIMATED AT 12.23 LBS/SQUARE FOOT PER IN OF DEPTH."

OR

(2)	"MAY UTILIZE BLAST FURNACE SLAG IN AN AGGREGRATE BLEND IN THIS ITEM IN ACCORDANCE WITH APPROVED MIX DESIGNS AND CURRENT SPECIFICATIONS. ESTIMATED AT 110 LBS/SQUARE YARD PER IN OF DEPTH." (DESIGN MEMO 8-89)

### DIVISION 700 - DRAINAGE, TRAFFIC AND ROADSIDE CONSTRUCTION

ADJUSTING DRAINAGE STRUCTURES

WHEN SPECIFYING THE ADJUSTING RING METHOD FOR ADJUSTMENTS IN HEIGHT WITHIN THE

LIMITATIONS OF THIS METHOD, 1 1/2" TO 3" (38 TO 75 MM), THE PLAN SHALL SHOW

THE FOLLOWING:

BID ITEM:  "ADJUSTING (NAME OF STRUCTURE, SIZE AND TYPE)"

REFERENCE THE BID ITEM AND SHOW THE FOLLOWING NOTE:

(1)	"RAISE CASTING IN ACCORDANCE WITH THE ADJUSTING RING METHOD DESCRIBED

IN SUBSECTION 710.03.03 OF THE STANDARD SPECIFICATIONS."

GUARDRAIL

REFERENCE ONE OF THE FOLLOWING NOTES TO THE TYPICAL SECTION ON PROJECTS WHERE

USING GUARDRAIL:

(2)	"WIDEN SHOULDER THREE  FEET WHERE INSTALLING GUARDRAIL."