This is in regards to adding AT&T or Cingular Bluetooth Phone support to a Surveyor Plus data collector from the Carlson Knowledge Base.

http://update.carlsonsw.com/kbase_main.php?action=main_page&cat=105

***I.  Check your OS Version***

To check what OS your Surveyor Plus data collector has:

1) Exit out of SurvCE

2) tap Start / tap Settings / tap the System tab in the bottom middle

3) scroll to the bottom of this screen and tap "System Config"

4) If your OS Build Version is 1.0.7 or higher you can follow the steps

below to add an AT&T_Cingular bluetooth phone connection.

!!!PLEASE NOTE: if your OS Build Version is 1.0.1 or 1.0.5 you MUST USE

Knowledge Base Article # 621 at the link at the bottom of the page FIRST

before any of the steps below will work !!!

If you are already running OS 1.0.7 or you have followed KB # 621 you can

proceed to the next step.

***II.  Setup a Bluetooth Cellphone Dial-up Icon***

This is a set of instructions for creating an internet connection through

an AT&T or Cingular Bluetooth Cellphone.

Turn on the Data Collector

1) Tap Start / Settings / Connections tab / Bluetooth

Turn on BT / Make this device discoverable

tap the Devices tab / tap "Add new device"

Follow the prompts to Bond to the phone with passkeys

Check the box for "Dialup Networking" as the Service / tap

FINISH

2) In the Connections Tab = tap the Connections icon

Under My ISP = tap "Add a new Modem Connection"

type in "Cingular" / pick Modem type of "Bluetooth"

tap Next / tap the Device from Step #1 / tap Next

3) Enter in the Phone Number exactly as shown below then tap Next

*99#

4)  Fill out the Username and Password as shown below then tap Finish

Username = WAP@CINGULARGPRS.COM

Password = CINGULAR1

Domain = leave it blank

***III. Installing Resco Registry Editor***

1) Click the Attachment at the bottom of this article and save the 2 files

"Resco_Regedit.exe" and "Cingular.reg" to your desktop or "My Documents"

folder on your computer

2) Hookup your Surveyor Plus with a USB cable to your Laptop and make sure

you are Connected with ActiveSync

3) Double-click the file "Resco_Regedit.exe" that you downloaded in Step #1

/ Click Run when it prompts you

4) The installation wizard for Resco Explorer 2008 will open up / click

Yes

5)  A screen showing a list of components will be shown = to save time

UNCHECK these boxes:

"FTP Add-in"

"Resco Today Plug-in"

"Desktop Shell Extension"

6) This should leave just "Reco Explorer 2008" and "Registry Add-in for

Resco" CHECKED / click Install

7) Click OK when it prompts you to Check your Mobile device / Click Finish

on your computer and click X to exit out of the registration screen

8) On the screen of the Surveyor Plus data collector tap the "Install"

button on the bottom left corner to install to the default location of

"Device" / tap the "Install" button a second time then when it says

Successfully Installed  tap OK in the upper right

9) Using ActiveSync on your Computer Copy the file "Cingular.reg" from your

Computer into the "My Documents" folder on the Surveyor Plus. {Cingular.reg

is listed as an Attachment at the bottom of this article}

10)  Tap the Start button / tap Programs / scroll down and tap "Resco

Registry" / tap OK to use "Touch Optimized"

11) Tap on "HKey_CURRENT_USER" / tap "Comm" / tap "RasBook"

12) At this point you should see this folderpath in your Status Bar at the

top left:

"\HKey_Current_User\Comm\RasBook”

13) Type the letter C on your keyboard and it should display a folder of

Cingular (if it does not then you have not followed Step # II above)

14) tap the Menu button in the bottom right / tap Import / tap on the file

"Cingular" that we copied over in Step # 9 to highlight it

15) It should say "Registry script file imported successfully" / tap OK /

tap X in the upper right twice

16) Hold down the On_Off button for a few seconds until a Menu pops up /

tap the Reset button to reboot the data collector

17) To test our new connection go into Start / Settings / Connections tab /

Connections Icon

18) Tap Manage Existing Connections / tap and HOLD "Cingular" / tap

Connect

the double arrows in the upper right start moving together

Returns you to the Connections screen

Exit back out to the Today Screen

19) Tap Start / Internet Explorer {or Start / Programs / Internet Explorer}

and type in google.com then hit the enter key to see if it loads up the

google webpage.  If it loads a webpage like google.com or carlsonsw.com

then you can tap X in the upper right and proceed to Knowledge Base Article

#472

***Please Note this is a ONE-TIME procedure that only needs to be done One

Time per data collector and it will not need to be repeated unless the

customer installs a new Operating System later on from Juniper

20) Use Article # 472 at the link on the bottom of this article to enter in

your SurvCE Settings