This is a rough set of instructions for manually creating a connection

using an Enfora GSM 0110 modem with AT&T_Cingular on an AllegroCX data

collector from the Carlson Knowledge Base.

http://update.carlsonsw.com/kbase_main.php?action=main_page&cat=105

!!!WARNING!!!  You will need an UNLIMITED data plan on your Cellphone

before using your cellphone to connect to the Internet for using it with

the GPS Reference networks !!!

PLEASE NOTE:  You will need an Enfora GSM 0110 Card with the PCMCIA Adapter

from Enfora and you may also need to purchase a Larger Plastic door for the

back of your AllegroCX data collector so it can close with the Enfora card

in place

If the steps below do not work properly - It is recommended you go to

www.junipersys.com and install the Latest Operating System on your

AllegroCX {i.e. - 1.07a}

1) Plug your Enfora GSM Card into the larger PCMCIA Adapter that came with

it

2) Plug the PCMCIA Card that has the Enfora card in it into the Slot under

the Back Cover of the AllegroCX by sliding it down into place until it is

tight

3) Start Menu / Settings / Network and Dialup / tap the File pull-down in

the Upper Left / tap New / type in ENFORA for the name / tap Next

4) Pick "Enfora-GSM0110" from the Pulldown List {If this is not displayed

in the list your Enfora Card may not be pushed in all the way}

6a) tap Configure / set the Baud Rate in the upper left to "115200"

6b) tap the Call options tab = uncheck the boxes for "Cancel call if not

connected" and "Wait for dial tone"

6c) Type in the code below into the "Insert Modem Commands" box

+cgdcont=1,,"wap.cingular"

then tap OK / tap OK when it prompts that the Baud rate has changed

!NOTE: This line of code is case sensitive and it needs to be entered in

exactly as shown above!

7) Tap Next / for the Phone Number enter in the code below then tap Finish

to return to the

Network and Dialup window

*99***1#

8a) double-tap the "ENFORA" icon we just created and fill in the User name

and Password

with the values below

User Name = WAP@CINGULARGPRS.COM

Password = CINGULAR1

check the box for "Save Password"

8b) tap "Dial Properties" / tap Edit in bottom right / Make sure there is

just the letter G in each of the 3 boxes {if there is different Letters or

symbols then delete the existing information and enter in an upper case G

for each box / and hit the ENTER key on keyboard  / tap OK}

!NOTE: if you tap the X in the upper right instead of hitting the Enter Key

the changes won't save

9) tap Connect

10)  It should go to a Status of "Connected" in a few moments with the

choice to Hide.  You can tap the Hide button.

At this point you should be connected to the Internet