SPECIAL NOTE FOR BEARING RETAINER

1.	DESCRIPTION.  Perform all work in accordance with the Kentucky Transportation Cabinet, Department of Highway’s Standard Specifications for Road and Bridge Construction and applicable Supplemental Specifications, the Standard Drawings (current editions), this note, and the attached detailed drawings for Bearing Retainer.   Section references are to the Standard Specifications.  This work consists of the following: (1) Furnish all labor, materials, tools, and equipment. (2) Bearing Retainer. (3) Any other work specified as part of this contract.

2.	MATERIALS.

- Structural Steel

ASTM Material, A709 Grade 50 Structural Steel Plates and Shapes.  Minimum structural steel yield strength Fy ~ 50,000 psi.

- Expansion Anchors

Expansion anchors shall be 3/4” diameter HILTI KWIK Bolt 3 SS or equal with a working shear capacity of 5.5 K minimum.

- Paint

See Section Special Note for Cleaning and Painting Structural Steel.

3.	CONSTRUCTION.

- Bearing Retainer.

Complete bearing retainer as specified in this special note and shown in the attached detailed drawings.

- Anchor Bolts.

See attached detailed drawings.

- Cleaning and Painting

All new steel shall be shop cleaned and receive the prime coat in accordance with Special Note for Cleaning and Painting Structural Steel.

- Verifying Field Conditions.

The Contractor shall field verify all plate and shape dimensions, bolt patterns and locations before ordering any material.  New material that is unsuitable due to variation in existing structure shall be replaced at the Contractors expense.

- Damage to the structure.

The Contractor shall bear all responsibility and expense for any and all damage to the structure during the repair work, even to the removal and replacement of a fallen span, should the fallen span result from the Contractors actions.

4.	MEASUREMENT.

- Bearing Retainer. The Department will measure the quantity as “Each” for Bearing Retainer”

5.	PAYMENT.

- Bearing Retainer (23166ED).  Payment at the contract unit price for “Each” is full compensation for furnishing and installing all material as specified.