SPECIAL NOTE FOR ONE STEP MEMBRANE

I.	DESCRIPTION

Perform all work in accordance with the Department's 2012 Standard Specifications, applicable Special Provisions, and Standard Drawings, except as hereafter specified. Article references are to the Standard Specifications. Furnish all materials, labor, equipment, tools, and incidentals necessary to complete the One Step Bridge Deck Waterproofing Membrane.

II.	MATERIALS

A.	Bridge Deck Waterproofing Membrane.  See Section 808.05.

B.	Surface Primer.  Use surface primer material recommended by the manufacturer.

C.	Asphalt Overlay.  Pave the bridge with the same type and thickness of asphalt that is being placed on the adjacent roadway.

III.	CONSTRUCTION METHODS

A.	Surface Preparation.  Thoroughly clean the entire surface area of the bridge deck prior to paving.  Clean by either air blasting or water blasting.  If water blasting is used, thoroughly dry area prior to placing the primer and membrane.

B.	Prime Coating.  Apply material to the bridge deck according to the Manufacturer's recommendations.

C.	One Step Membrane.  Apply the membrane to the entire surface of the bridge deck. Overlap the membrane a minimum of 1 foot onto the adjacent roadway approaches.  Extend the membrane up the face of the curbs a minimum of 1½ inches.  Overlap membrane as necessary to achieve 100% coverage of the bridge deck.  Install as recommended by the manufacturer, straight and wrinkle free, with no curled or uplifted edges.

E.	Pave Bridge with Asphalt Surface.  Pave the bridge with asphalt overlay immediately after the one step membrane is in place.

IV.	METHOD OF MEASUREMENT

A.	One Step Membrane.  The Department will measure this item in square yards.  The one step membrane area (A) will be calculated from the (L) length of the bridge plus 1 foot on each approach multiplied by the (W) width of the bridge curb to curb. A=((L+2) X W)/9.  This calculated area will be the area measured for payment; laps, cutoffs and waste will not be measured for payment.

V.	METHOD OF PAYMENT

A.	Waterproofing Membrane.  Payment at the contract unit price per square yard shall be full compensation for all labor, equipment, materials and incidentals for surface preparation and furnishing and installing the surface primer and the one step membrane according to the Manufacturer's recommendations and these notes.

WATERPROOFING MEMBRANE

04/11/2019