Special Note for Signing

- DESCRIPTION

Except as provided herein, this work shall be performed in accordance with the current edition of the Manual on Uniform Traffic Control Devices (MUTCD), the Department's current Standard Specifications and Interim Supplemental Specifications, applicable Standard and Sepia Drawings, and applicable Special Provisions. Article references are to the Standard Specifications.  This project shall consist of furnishing all labor, equipment, materials, and incidentals for the following:

(1) Maintaining and Controlling Traffic; (2) Furnish, Fabricate, and Erect Signs; and (3) All other work specified in the Contract.

- MATERIALS

All materials shall be sampled and tested in accordance with the Department's Sampling Manual and the materials shall be available for sampling a sufficient time in advance of the use of the materials to allow for the necessary time for testing unless otherwise specified in these Notes.

- Maintain and Control Traffic.  See Traffic Control Plan.

- Erosion Control.  See Special Note for Erosion Control.

- CONSTRUCTION METHODS

- Maintain and Control Traffic.  See Traffic Control Plan.

- Site Preparation.  Be responsible for all site preparation including, but not limited to: clearing and grubbing, staking, excavation, backfill, and removal of obstructions or any other material not covered by other items.  Perform all site preparation only as approved or directed by the Engineer.

- Staking.  See Special Note for Staking.

- Signs and Posts.  Before beginning installation, the Contractor shall furnish to the Engineer drawings, descriptions, manufacturer’s cuts, etc. describing and/or detailing all material to be used.  Mill test reports for beams, steel panels, and each different gauge of aluminum or steel sheeting used must be submitted to the Division of Construction and approved prior to erection.

Fabricate sheet signs from .080 or .125 gauge aluminum alloy 5052-H38 or 6061-T6, in accordance with ASTM B-209, and to the size and shape specified.  Prepare the side of the aluminum sheet to receive the retroreflective background material according to the recommendations of the sheeting and retroreflective material manufacturer(s).  Sheeting used as background material for sign faces is to be the color specified and visually in accordance with the standard requirements of ASTM D-4956 and meet the requirements of Section 830 of the Standard Specifications.  Contrary to Section 830.02.06, only the types and colors of sheeting as specified in the proposal will be accepted.  All retroreflective material shall be fabricated and assembled in accordance with the specifications and/or recommendations of the manufacturer(s).

All hardware for the erection of sheeting signs shall be rust resistant:  stainless steel, zinc coated, aluminum, or an Engineer approved material.  All beams and sign posts shall be of sufficient lengths so that a single, continuous length of sign post extends from the top of the sign to the required embedment in the anchor.  Splicing of the sign post shall NOT be allowed.  For installations in soil, Type I steel posts shall be mounted on either a standard anchor, with soil stabilizer plate, or on a Type D breakaway sign support.  Refer to Sheeting Sign Detail Sheet 1 of 2 for installation details for a standard anchor with soil stabilizer plate.  When installing a standard anchor with soil stabilizer plate, if solid rock is encountered, the Contractor shall drill a hole to the required depth into the rock, install the anchor into the hole, and backfill the anchor post with concrete, or other method approved by the Engineer.  The cost shall be incidental to Type I steel post, and a soil stabilizer plate will not be required.  Refer to Standard Drawing RGX-065, current edition, for installation details of Type D breakaway sign supports.  Approved manufacturers for Type D breakaway sign supports have been placed on the list of approved materials. For installations to be installed on sufficiently cured existing concrete, such as a sidewalk, concrete median, etc., Type I steel posts shall be mounted on a Type D Surface Mount.  For Type D Surface Mounts use only Redi-Torque Model 280 Surface Mount Slip Base Assembly (part number SMSB) by Xcessories Squared of Auburn, IL. Prior to installation, the Contractor shall submit to the Engineer shop drawings of the Type D Surface Mount.  Install the Type D Surface Mount according to all the applicable requirements of the manufacturer (see shop drawings).  During installation of the Type D Surface Mount, if the existing concrete surface is not flat (i.e. has a slope), galvanized metal shims may be required to ensure the surface mount base is level and plumb.  If a Type D breakaway sign support is specified for a location that has an asphalt surface, the Contractor shall install the Type D breakaway sign support detailed on Standard Drawing RGX-065, current edition. All steel post shall meet the requirements of Section 832.  All hardware including, but not limited to, sign post anchors, soil stabilizer plates, nuts, bolts, washers, fasteners, fittings, shims, and bracing, or any other incidentals necessary to erect the signs shall be furnished by the Contractor and will be incidental to the work.

New concrete bases, posts, support anchors, signs, etc. are to be installed prior to dismantling any existing sign(s).  The removal of existing signs, posts, and support anchors is to be performed concurrently with the installation of new signs, posts, and support anchors, under the same lane closure during the same work shift.  Completely remove existing sign support anchors or remove them to a minimum depth of six (6) inches below existing ground line and backfill the disturbed area to the existing ground line.

When listed on the plans and/or summaries, fabricate Reflective Sign Post Panels from .080 gauge aluminum alloy 5052-H38 or 6061-T6, in accordance with ASTM B-209 and to the size(s) specified.  Prepare the side of the aluminum sheet to receive the retroreflective background material according to the recommendations of the sheeting and retroreflective material manufacturer(s).  Sheeting for the Reflective Sign Post Panels shall be the same Type and color as the sign installed on the post.  Examples include:

  - Red, fluorescent yellow, and fluorescent yellow-green (Type XI Sheeting)

  - White and yellow (Type XI Sheeting)

Reflective Sign Post Panels shall be 2 inches wide and will typically have a height of 60 inches for rural installations and typically have a height of 84 inches for urban installations.  There will be certain instances where a proposed Reflective Sign Post Panel will have a height dimension less than 60 inches; typically, this will be when the bottom of the bottom-most sign is mounted lower than the standard 5 ft minimum mounting height (e.g. 3 ft or 4 ft mount heights).  In those cases, the height of the Reflective Sign Post Panel is expected to closely match (within 1-2 inches) the distance between the top of the anchor or support to the bottom edge of the bottom-most sign.  Reflective Sign Post Panels shall have three 3/8” holes (one hole in the top 3”, one hole near the center, and one hole in the bottom 3”) that align with the holes on the Type I steel post.

All manufactured sheeting signs shall be free of visual defects including, but not limited to:  cracks, tears, ridges, humps, discoloration, etc., and defective signs shall be replaced at no additional cost to the Department.

All sign blanks shall be hole punched by the manufacturer for either horizontal or vertical installation.  Attach all aluminum sheeting signs to square post with 3/8” all steel rivets and nylon washers.  Use bracing as indicated on the plans, summaries, and/or standard signing detail sheets, and/or when directed by the Engineer and/or District Traffic Engineer.

All sign posts shall be attached to anchors with 5/16” corner bolts and 5/16” flanged nuts, and all post and anchor cuts shall be treated with a Cold Galvanizing Compound spray.

Sign posts shall be erected vertically by using a bubble level.  The tolerance shall be a two (2) degree angle in any direction.  For locations where more than one sign is mounted beside each other, the posts shall be spaced to provide approximately six inches (6”) of spacing between signs.

- Remove & Relocate Sheet Signs.  When listed on the plans and/or summaries, and/or as directed by the Engineer and/or District Traffic Engineer, remove the specified existing sheet sign(s) from the existing post(s) and reinstall on a new sign post.  Once the specified existing sheet sign(s) have been removed and relocated, and if the existing sign post(s) are no longer needed to support other existing signs, removal of the existing sign post(s) will be paid under the bid item REMOVE SIGN.  If any of the existing hardware components (bracing, brackets, bolts, rivets, etc.) are found to have pre-existing damage or are damaged during the Contractor’s removal and reinstallation efforts, the Contractor shall provide the necessary replacement hardware for proper re-installation of the sheet sign.  These components shall be incidental to the bid item REMOVE AND RELOCATE SHEET SIGNS.

Prior to removing and reinstalling a sheet sign, the Contractor shall first review the existing sheet sign for damage.  It is the Contractor’s responsibility to notify the Engineer of any existing sheet sign damage prior to removal and relocation of the sheet sign, so that it can be documented that the existing sheet sign had pre-existing damage.  If the Contractor does not make the Engineer aware of pre-existing damage prior to detaching the sheet sign from its existing post, the Department will assume the damage was the result of the Contractor’s removal and reinstallation efforts.  The Contractor shall replace any sheet signs that are damaged during the removal and reinstallation efforts.  Replacement of sheet signs damaged by the Contractor shall be incidental to the bid item REMOVE AND RELOCATE SHEET SIGNS.

If the existing sheet sign is found to have pre-existing damage, the Department will provide the Contractor with a new sheet sign to replace the sheet sign with pre-existing damage.  Detaching the existing, damaged sheet sign from the existing post and attaching the new, Department-provided sheet sign to the new sign post shall be incidental the bid item REMOVE AND RELOCATE SHEET SIGNS.

- Remove & Relocate Sign Assemblies.  When listed on the plans and/or summaries, and/or as directed by the Engineer and/or District Traffic Engineer, remove the specified existing sign assemblies from the existing location and reinstall in a new location.  The Department will consider all signs attached to one or more connected posts as a single sign assembly, no matter how many signs are attached to the existing sign assembly.  If any of the existing hardware components (bracing, brackets, bolts, rivets, etc.) are found to have pre-existing damage or are damaged during the Contractor’s removal and reinstallation efforts, the Contractor shall provide the necessary replacement hardware for proper re-installation of the sign assembly.  These components shall be incidental to the bid item REMOVE AND RELOCATE SIGN ASSEMBLY.

Prior to removing and relocating a sign assembly, the Contractor shall review the existing sign(s) and sign post(s) for damage.  It is the Contractor’s responsibility to notify the Engineer of any sign or sign post damage prior to removal and relocation of the sign assembly, so that it can be documented that the existing sign and/or sign post had pre-existing damage.  If the Contractor does not make the Department aware of pre-existing damage prior to removing a sign assembly from its existing location, the Department will assume the damage was the result of the Contractor’s removal and reinstallation efforts.  The Contractor shall replace any components of a sign assembly that are damaged during removal and relocation. Replacement of any components damaged by the Contractor shall be incidental to the bid item REMOVE AND RELOCATE SIGN ASSEMBLY.

If an existing sign that is part of a sign assembly to be removed and relocated is found to have pre-existing damage, the Department will provide the Contractor with a new sign to replace the sign with pre-existing damage.  Detaching the existing, damaged sign from the existing post and attaching the new, Department-provided sign to the relocated existing post shall be incidental to the bid item REMOVE AND RELOCATE SIGN ASSEMBLY.

If an existing sign assembly that is to be removed and relocated is found to not have an existing soil stabilizer plate, or if the soil stabilizer plate and/or anchor is damaged during removal, then a new soil stabilizer plate and/or anchor shall be provided by the Contractor and shall be incidental to the bid item REMOVE AND RELOCATE SIGN ASSEMBLY.

If an existing sign assembly that is being relocated is not currently mounted on a Type D breakaway sign support, but the plans and/or summaries indicate, or wind load standards dictate, a Type D breakaway sign support or a Type D Surface Mount is required, provide and install the specified Type D support as part of the removal and reinstallation efforts.  Type D breakaway sign supports shall be paid under the bid item GMSS TYPE D and Type D Surface Mount supports shall be paid under the bid item GMSS TYPE D (SURFACE MOUNT).

If an existing sign that is being relocated is found to have pre-existing damage to one or more of the sign post, the Department will NOT utilize the bid item REMOVE AND RELOCATE SIGN ASSEMBLY for removing and relocating such a sign assembly.  Instead, the Department will require the Contractor to install a new sign post(s) at the new location, and pay for the new post(s) under the bid item STEEL POST TYPE I.  Detaching the existing sign(s) from the existing, damaged post(s) and attaching the existing sign(s) to the new sign post(s) shall be incidental to the bid item STEEL POST TYPE I.  Any hardware that is needed to complete the installation shall also be incidental to the bid item STEEL POST TYPE I.  Removal of the existing damaged post(s) and any other sign components not needed will be paid under the bid item REMOVE SIGN.

- Property Damage.  The Contractor shall be responsible for all damage to public and/or private property resulting from the Contractor’s activities.  Repair or replace damaged roadway features in like kind materials and design as directed by the Engineer at no additional cost to the Department.  Repair or replace damaged private property in like kind materials and design to the satisfaction of the owner and the Engineer at no additional cost to the Department.

- Coordination with Utility Companies.  Locate all underground, above ground, and overhead utilities prior to beginning construction.  Be responsible for contacting and maintaining liaison with all utility companies that have utilities located within the project limits.  Do not disturb existing overhead or underground utilities.  It is not anticipated that any utility facilities will need to be relocated and/or adjusted; however, in the event that it is discovered that the work does require that utilities be relocated and/or adjusted, the utility companies will work concurrently with the Contractor while relocating their facilities.  Be responsible for repairing all utility damage that occurs due to the Contractor’s operations at no additional cost to the Department.  NOTIFY THE ENGINEER AND THE UTILITY OWNER(S) IMMEDIATELY WHEN IT IS DISCOVERED OR ANTICIPATED THAT ANY UTILITY CONFLICT COULD DELAY THE CONTRACTOR’S OPERATIONS.  If the total delay exceeds ten working days, an extension of the specified completion date will be negotiated with the Contractor for delay to the Contractor's work; however, no extension will be granted for any delay caused by the Contractor’s failure to notify the Engineer and/or the utility company as specified above when a conflict is discovered or anticipated as specified.

- Caution.  The information in this proposal and the type of work listed herein are approximate only and are not to be taken as an exact evaluation of the materials and conditions to be encountered during construction; the bidder must draw his/her own conclusions when developing the Unit Bid Prices for each bid item.  As such, if the conditions encountered are not in accordance with the information shown, the Department does not guarantee any changes to the Unit Bid Prices nor extension of the contract will be considered.  The Department will pay for bid item quantity overruns, but only if pre-approved by the Engineer.

- Control.  Perform all work under the absolute control of the Department.  Obtain the Engineer’s approval of all designs required to be furnished by the Contractor prior to incorporation into the work.  The Department reserves the right to have other work performed by other contractors and its own forces, and to permit public utility companies and others to do work during the construction within the limits of, or adjacent to, the project.  Conduct operations and cooperate with such other parties so that interference with such other work will be reduced to a minimum.  The Department will not honor any claims for money or time extension created by the operations of such other parties.

Should a difference of opinion arise as to the rights of the Contractor and others working within the limits of, or adjacent to, the project, the Engineer will decide as to the respective rights of the various parties involved in order to assure the completion of the Department’s work in general harmony and in a satisfactory manner, and the Engineer’s decision shall be final and binding upon the Contractor.

- Clean Up, Disposal of Waste.  Clean up the project area as work progresses.  Dispose of all removed concrete, debris, and other waste as per Section 204.03.08.  The Department will incur no cost to obtain the disposal sites.  The Department will NOT make direct payment for disposal of waste and debris from the project.  Existing anchors, signs, posts, and any other hardware or material removed from the site are to become the property of the Contractor.  See Special Provision for Waste and Borrow Sites.

- Final Dressing, Seeding and Protection.  Grade all disturbed areas to blend with the adjacent roadways features and to provide a suitable seed bed.  Apply Class A Final Dressing to all disturbed areas, both on and off the Right-of-Way.  Sow all disturbed earthen areas with the applicable seed mixture(s) according to Section 212.03.03.

- Erosion Control.  See Special Note for Erosion Control.

- METHOD OF MEASUREMENT

- Maintain and Control Traffic.  See Traffic Control Plan.

- Site Preparation.  Other than the bid items listed, the Department will NOT measure Site Preparation for payment, but shall be incidental to the project bid items.

- Signs and Reflective Sign Post Panels.  The Department will measure the finished in-place area of signs in Square Feet.

- Sign Posts.  The Department will measure the finished in-place length of sign posts in Linear Feet, from the top of the anchor, or top of the sign support, to the top of the sign post.  Laps, cutoffs, excess, and waste will NOT be measured for payment.

- Type D Breakaway Sign Supports.  The Department will measure Type D breakaway sign supports as Each support installed.

- Type D Surface Mounts.  The Department will measure Type D Surface Mounts as Each surface mount installed.

- Class A Concrete for Signs.  The Department will measure the Class A Concrete used in conjunction with Type D breakaway sign support installations in Cubic Yards.  Any concrete that is required as backfill due to hitting rock during a standard installation shall be incidental to the bid item Steel Post Type I, and soil stabilizers will not be required.

- Clean Up, Disposal of Waste, Final Dressing, Seeding and Protection.  The Department will NOT measure for payment the following activities: Clean Up, Disposal of Waste, and Final Dressing.  These activities shall be incidental. Seeding and Protection shall be measured according to Section 212.

- Erosion Control.  See Special Note for Erosion Control.

- Remove Sign.  The Department will consider all signs attached to one or more connected posts as a single sign.  The Department will measure as Each sign assembly removed and NOT each individual sign removed.

- Remove & Relocate Sheet Signs.  The Department will measure sheet signs removed from an existing sign post and reinstalled on a new sign post as Each sheet sign removed and reinstalled. as indicated in the contract documents, or as directed by the Engineer.  The new sign post shall be measured as indicated in paragraph D. of this section.

- Remove & Relocate Sign Assemblies.  The Department will consider all signs attached to one or more connected posts as a single sign assembly.  When the contract documents indicate that an existing sign assembly is to be removed from its existing location and reinstalled in a new location, the Department will measure and pay for “Remove and Relocate Sign Assembly” as each sign assembly removed and relocated; NOT each individual sign removed and relocated.

- Items Provided by KYTC.  The Department will NOT measure for payment the installation of signs and/or surface mounts provided by KYTC.  These activities shall be incidental to the bid item Steel Post Type I.

- BASIS OF PAYMENT

- Maintain and Control Traffic.  See Traffic Control Plan.

- Signs and Reflective Sign Post Panels.  The Department will make payment for the completed and accepted quantities under the bid item SBM ALUM SHEET SIGNS .125 IN or .080 IN.  The Department will consider payment full compensation for all work and incidentals necessary to install the signs, as required by these notes and the details found elsewhere in the plans/proposal, at the locations indicated on the summary sheets, plans, and/or as directed by the Engineer.

- Sign Posts.  The Department will make payment for the completed and accepted quantities under the bid item STEEL POST TYPE I.  The Department will consider payment full compensation for all work and incidentals necessary to install the sign posts as required by these notes and the details found elsewhere in the plans/proposal.

- Type D Breakaway Sign Supports.  The Department will make payment for the completed and accepted quantities under the bid item GMSS TYPE D.  The Department will consider payment full compensation for all work and incidentals necessary to install the Type D breakaway sign supports as required by Standard Drawing RGX-065, current edition.

- Type D Surface Mounts.  The Department will make payment for the completed and accepted quantities under the bid item GMSS TYPE D (SURFACE MOUNT).  The Department will consider payment full compensation for all work and incidentals necessary to install the Type D surface mounts according to all applicable manufacturer requirements.

NOTE:  The permissible Type D Surface Mount alternative is:  Redi-Torque Model 280 Surface Mount Slip Base Assembly (part number SMSB) by Xcessories Squared of Auburn, IL.

- Class A Concrete for Signs.  The Department will make payment for the completed and accepted quantities, used in conjunction with Type D breakaway sign support installations, under the bid item CLASS A CONCRETE FOR SIGNS.  The Department will consider payment full compensation for all work and incidentals necessary to install the concrete as required by Standard Drawing RGX-065, current edition.

- Remove Sign.  The Department will make payment for the completed and accepted quantities under the bid item Remove Sign.  The Department will consider payment full compensation for all work and incidentals necessary to remove the existing signs, posts, anchors, and any other sign material or hardware, from the locations indicated on the summary sheets, plans, and/or as directed by the Engineer.

- Remove & Relocate Sheet Signs.  The Department will make payment for the completed and accepted quantities under the bid item REMOVE AND RELOCATE SHEET SIGNS.  Any hardware that is needed to complete the removal and reinstallation shall be incidental.  The Department will consider payment full compensation for all work and incidentals necessary to remove and reinstall the existing sheet signs as indicated on the plans, summaries, and/or as directed by the Engineer.

- Remove & Relocate Sign Assemblies.  The Department will make payment for the completed and accepted quantities under the bid item REMOVE AND RELOCATE SIGN aSSEMBLY.  Any hardware that is needed to complete the removal and reinstallation shall be incidental.  The Department will consider payment full compensation for all work and incidentals necessary to remove and reinstall the existing sign assembly as indicated on the plans, summaries, and/or as directed by the Engineer

- Erosion Control.  See Special Note for Erosion Control.